The Network

The Brumhadinho max time series is a set of 21 time series corresponding to 21 (of the 220) pixels that had the maximum displacement at least once during the monitoring of the tailings dam. We created two separate adjacent matrices for these time series.

Network 1: Max count net

To construct the network we compute pairwise Spearman similarity between the 21 time series based on the cumulative count of the number of times a pixel has been maximum. Then we classify two pixels to be connected (1) if the similarity (spearman’s correlation) is greater than 0.8, else 0.

Network 2: Max spectral net

The spectral net is constructed using pairwise Spearman’s periodogram correlation between magnitudes of all displacement time series after removing the linear trend. Two pixels are considered connected (1) if the magnitude of the periodogram Spearman correlation >0.4.

Network 3: Vario net

For each day we compute the spatial “Cressie” (robust), directional (anisotropic) variogram for displacement, at pairwise distances. We adjust the variogram computation for trend surface (coordinates) and temporal trend – previous day’s displacement signal. We plot the marginal distribution of variograms at all distances over the 44 days. We censor the network to distances at which the marginal distribution stabilizes, which for this data is 50 units. That is a pair of pixels are considered connected by an edge (1) if their Euclidean distance is less than 90 units else they are not connected. Like the other networks this logic may have its own limitations.